"A-201":

AI Summary

"Summary": "Shortened staff elevation; updated bale facade; added stair tower.",

- "Deleted Artist’s Studio elevation; shortened Staff Break Room segment",

- "Updated Bale Storage façade dimensions (+742 SF) and related finish notes",

- "Placed new exterior stair tower with guardrails in North Elevation",

- "Added genset exhaust louver in East Elevation",

- "Adjusted height callouts and material keynotes to match revised building areas"

Human Summary:

Legend updated:

Removed

Solid wall panel

Metal Wall Panel

Louver/vent

CMU Wall.

Added

BUTLERIB II METAL WALL PANELMETAL SALES METALLIC SILVER (K7)

BUTLERIB II METAL WALL PANEL METAL SALES MISTIQUE PLUS (W31)

2-HOUR FIRE-RATED PANEL MATCH METAL SALES METALLIC SILVER (K7)

2-HOUR FIRE-RATED PANEL MATCH METAL SALES MISTIQUE PLUS (W31)

GALV. SHEET METAL TRIM "ATHENS RED" PANTONE 185C

POLYCARBONATE OR BUTLERIB II WALL LITE PANEL, SEE ENLARGED ELEVATIONS

LOUVER/VENT

CMU WALL

Building Elevation Sketch Updated List:

North Elevation / Pendelton Street Elevation

Building Stretched - Grid A to E Dimensions increased 248’-1” to 271’-0” and Grid A to D Dimensions increased 148’-4½” to 171’-3½” & (Edge of Fire Panel - 43’-6½“ + 104’-10”) to 171’-3½”.

T.O. CMU Wall Height Reduced 11’-0” to 10’-0” at only the Length of 19’-10” (Dimensions Added).

Eave Height Reduced 40’-0” to 36’-113/8” (Breakup Dimensions Added).

Added Detail 6A/A-502 and Notification tag at Grid A.

Added Detail 11/A-505 & 3/A-503 and Added Notification tag.

Added Detail 10/A-502 and Notification tag.

Removed Downspouts Drain to BMPS and Removed Notification Arrow & Text.

Added Typ.Gaurdrail.42” Above Adjacent Roof Surface, Shown & Mentioned in Roof Plan A-131.

Added 2 HR Fire Rated Panel and Added Notification Arrow & Text.

Changed in Detail 10/A-502 to 8/A-502 and Changed Notification 10/A-502 to 8/A-502.

Removed Detail 10/A-502.

Fire Rated Panel & Metal Panel added .

Galv.Sheet Metal Trim “Athens Red” Pantone 185C Fixed above the Parapet (Existing Building) at the Level of 28’-2” From 27’-0” (Height of 1’-2”).

Staircase fixed & Connected with Roof Ramp.

D13 Opening shifted and demarked as D7 Dimensions Changed.

Additional Openings Added D8.2, D8.1 & D8.

CMU Wall length & Height size changed Refer Detail

Removed Wall Section Detail WS-3/A-303.

Details added Ref.3B/A-502, 9/A-502, 3B/A-502, 2/A-502, N/A-203, 2/A-511, 3/A-502, 11/A-505, 3/A-407, 2/502, 3B/A-502, 9/A-502, 9/A-502, 3B/A-502, 9/A-502 & 2/A-502.

Opening Detail Changed 10/A-502 to 7/A-502 at D2

Detail Changed 1/A-207 to 1/A-208.

Details added 8/A-505, 8/A-505 & 1/A-503.

Detail Changed 3/A-207 to 3/A-208 .

Details Added 1/A-503 & 11/A-503.

Added Pump room & Rest Room.

East Elevation / De Garmo Ave Elevation

CMU Wall Height noted up & down.

Louver Detail to be checked and Detail Changed 5/A-207 to 5/A-208.

Wall Cladding changed from Acrylic wall Panel to Polycarbonate wall Panel.

Knock out Panel Removed.

Opening D18 to D11, D15 to D9 and D7 to D5.1 changed and Added D22.

South Elevation / Randall Street Elevation

Detail Changed 1/A-208 to 1/A-209 and 3/A-208 to 3/A-209.

Opening D23 Shifted.

West Elevation

Gutter Detail Changed 2/A-208 to 2/A-209 and 2/A-207 to 2/A-208.

Dimensions to be checked with ref code.