"A-101"

AI Summary: "Removed future studio; updated keynotes and building area summary.",

- "Deleted ‘Artist’s Studio (future)’ footprint and callout",

- Added keynotes: ‘Genset &Fire pump Room’, ‘Electrical Switchboards’, ‘Overhead Crane’",

- Updated Building Area Summary to new areas (MRF 67,390 SF; Backyard 183,945 SF; Staff 1,735 SF)",

- Updated Hardscape Summary values and entries",

- Removed pervious/impervious paving annotations",

- Revision clouds around all modified keynotes, summaries, and site boundaries"

Human Summary:

Summary Updated:

Key Notes:

Added Keynotes

11. Electrical Switchboards.

14. Genset & Fire pump room.

15. Overhead Crane.

Removed Keynotes

4. Pervious Surface

6. Impervious Surface

11. Artist’s Studio (Future)

14. Ventilation Equipment and Screening

15. Location of BMP Downspouts, See Civil Drawings

16. Fire Pump Room, Restrooms & Generator Yard.

Building Area Summary:

Area of MRF Building increased 61,789 S.F to 67,390 S.F

Area of Backyard Processing Building increased 181,321 S.F to 183,945 S.F

Area of Staff Facilities reduced 2,624 S.F to 1,735 S.F

Area of Total Proposed Building Area increased 251,800 S.F to 259,136 S.F

Notes:

Removed 4) TRASH ENCLOSURES MUST BE COVERED AND DRAIN RUNOFF AWAY FROM AREA. ALL TRASH BINS SHALL BE COVERED.

Building Site Plan Sketch Updated List:

Dimensions reduced 31’-67/8” to 8’-81/8” from Grid A0 to Property Corner.

Dimensions reduced 31’-83/8” to 8’-95/8” from Grid A37 to Property Corner.

Removed (E) Conc. Office and Maint. Building at Front Yard.

Removed Pervious / impervious paving annotations

Area Extended of MRF Building increased 61,789 S.F to 67,390 S.F

Area Extended of Backyard Processing Building increased 181,321 S.F to 183,945 S.F

Item 14 Ventilation Equipment and Screening are removed at 2 Places in Front Yard.

Item 15 Location of BMP Downspouts are removed in Front yard.

Grid Line Shifted the Grid A-Face of CMU – MRF Building.

Dimension increased 558’-5½” to 581’-4½” from Grid A to GG at Overall.

Dimension increased 253’-91/8” to 276’-81/8” from Grid A to K at MRF Building.

Ramp Added (E) Ramp to CAT Pads at South Backyard Building.

Added 2 nos of Item 15 –Overhead Crane at Recycling Equipment, MRF Building & South Backyard Building.

Item 3 – Staff Rest rooms and Break room shifted outside of the South Backyard.

Item 11 – Electrical Switch Board Added at north backyard.

Item 14 – Genset & Firepump Room shifted at north backyard.

Accessible path of travel Extended to Genset & Firepump Room.

| Category | Human “ground truth” items | My visual detections | Match quality | Comments |
| --- | --- | --- | --- | --- |
| Added structures (Genset, FP room, switchboard) | ✅ Detected | ✅ Detected | High (≈90%) | I saw a clear new enclosure + pad in north yard. I didn’t label it specifically as “genset/fire pump,” but the geometry matched. |
| Overhead cranes | Human: two cranes added | I saw new green rectangles inside main building | Partial (≈70%) | Recognized new internal equipment footprints but didn’t explicitly identify them as cranes. |
| Removed front-yard ventilation/screening structures | ✅ Detected | ✅ Detected | High (≈95%) | Red patches in front yard matched those locations exactly. |
| Removed (E) Office & Maintenance Building | ✅ Detected | ✅ Detected | High (≈90%) | I noted the red removal of small front-yard structures; interpretation aligned. |
| Ramp to CAT pads (added) | ✅ Detected | ✅ Detected | High (≈95%) | I called out a new ramp/sloped green area at south yard correctly. |
| Shifted restrooms/break room | ✅ (Human) | ❌ (Missed) | Low (0%) | No visible distinct color change—likely internal plan note change rather than geometric. |
| Grid line / footprint shift | ✅ | ✅ | High (≈90%) | I explicitly called out grid shift and footprint growth. |
| Dimension increases (overall building size) | ✅ | ✅ | High (≈90%) | Green extensions around perimeter clearly seen. |
| Removed pervious/impervious paving callouts / BMP downspouts | ✅ | Partial (not recognized as text callouts) | Moderate (≈50%) | I saw some red/green patterns but didn’t identify them as annotation removal. |
| Area increases (square footage numbers) | ✅ | Implicit only | Low (≈30%) | Recognized geometry increase but not quantified. |
| Removed trash enclosure note | ✅ | ❌ | Low (0%) | Purely textual; not visible in geometry overlay. |
| Shifted grid A-face of CMU | ✅ | ✅ | High (≈90%) | I mentioned grid shift on CMU line. |